SOC

It’s Stand for System on chip.

In an SoC, a crossbar (often called interconnect crossbar or AXI crossbar) is the central switching fabric that connects multiple masters to multiple slaves and allows parallel data transfers.

Think of it as the traffic junction of the SoC.

🔹 Simple Definition

Crossbar = hardware switch that routes transactions from any master to any slave based on address and rules.

🔹 Why Crossbar is Needed in SoC

In an SoC you have:

- Multiple masters
CPU, DMA, GPU, PCIe controller, etc.

- Multiple slaves
DDR, SRAM, UART, GPIO, registers

Without a crossbar:

- Only one master can talk at a time

- Performance collapses

🔹 Real-World Analogy 🚦

Imagine a road junction:

- Roads coming in = masters

- Roads going out = slaves

- Traffic lights + routing = crossbar logic

The crossbar decides:

- Who goes first

- Where each vehicle goes

- How many can go in parallel

🔹 Crossbar in AXI-Based SoC

Most modern SoCs use an AXI crossbar.

AXI Crossbar Responsibilities

- Address decoding

  - Decide which slave an address belongs to

- Routing

  - Connect master → correct slave

- Arbitration

  - Multiple masters want same slave → who wins?

- ID handling

  - Keep transactions matched (out-of-order)

- Parallelism

  - Master A → Slave X

  - Master B → Slave Y
(at the same time!)

🔹 Basic Crossbar Diagram

CPU ----\

\

DMA -----+---- AXI CROSSBAR ---- DDR

|                    \

GPU ----/                      \-- SRAM

\-- UART

🔹 Crossbar vs Bus (Very Important)

| Feature | Bus | Crossbar |
| --- | --- | --- |
| Parallel access | ❌ | ✅ |
| Scalability | Poor | Good |
| Performance | Low | High |
| Modern SoCs | ❌ | ✅ |

Old SoCs used shared bus.
Modern SoCs use crossbars / NoCs.

🔹 Crossbar vs NoC

| Crossbar | NoC |
| --- | --- |
| Simple | Complex |
| Good for small SoCs | Used in big SoCs |
| Limited scaling | Highly scalable |

📌 Many SoCs:

- Small: AXI Crossbar

- Large: Network-on-Chip (NoC)

🔹 What Happens During a Transaction?

Example: CPU writes to UART register

- CPU sends AXI write address

- Crossbar:

  - Decodes address

  - Selects UART slave

- If DMA also wants UART:

  - Arbitration logic chooses one

- Data is routed

- Response is routed back

🔹 Crossbar in Verification (Your Area 💡)

What SoC DV Engineers Check

- Correct address routing

- Arbitration fairness

- No deadlock

- Ordering per AXI rules

- Reset behavior

- Backpressure handling

Common Bugs

- Wrong address decode

- Starvation of low-priority master

- ID corruption

- Response routed to wrong master

🔹 PCIe Analogy (Easy for You)

| PCIe | SoC Crossbar |
| --- | --- |
| Switch | AXI Crossbar |
| TLP routing | Address routing |
| Tag tracking | AXI ID tracking |
| Arbitration | VC / scheduler |

🔹 Interview One-Liner 🎯

“A crossbar in SoC is an interconnect fabric that connects multiple masters to multiple slaves, enabling parallel AXI transactions with address-based routing and arbitration.”

If you want, next I can explain:

- AXI crossbar vs AXI interconnect

- How arbitration works internally

- Crossbar UVM testcases

- Real silicon crossbar bugs

Just tell me 👍

This is a very good SoC question 👍
People often use AXI crossbar and AXI interconnect interchangeably, but they are not exactly the same.

I’ll explain this clearly, practically, and interview-ready, with your PCIe / UVM background in mind.

🔹 Short Answer (Interview Version)

AXI crossbar is a type of AXI interconnect.
Crossbar focuses on direct master–slave switching, while interconnect is a broader term that may include crossbars, arbiters, bridges, and NoC logic.

🔹 AXI CROSSBAR

What it is

An AXI crossbar is a fully connected switch fabric:

- Any master → any slave

- Direct, parallel paths

Structure

- Master ports × Slave ports

- Dedicated routing paths

M0 ─┬───────── S0

M1 ─┼─ AXI ─── S1

M2 ─┴───────── S2

Key Characteristics

✅ High performance
✅ True parallelism
❌ Area & power expensive
❌ Poor scalability (N×M growth)

When Crossbar is Used

- Small to medium SoCs

- Performance-critical paths

- Few masters/slaves

Example:

- CPU + DMA → DDR

- CPU → peripherals

Verification Focus

- Arbitration per slave

- ID preservation

- No starvation

- Ordering rules

📌 PCIe analogy:
AXI crossbar ≈ PCIe switch (small scale)

🔹 AXI INTERCONNECT

What it is

AXI interconnect is a generic term for any fabric that connects AXI masters and slaves.

It may include:

- Crossbars

- Shared buses

- Arbiters

- Bridges (AXI-to-APB)

- Clock crossing logic

- Firewalls

- NoC routers

Types of AXI Interconnect

- Crossbar-based

- Bus-based

- Hierarchical interconnect

- Network-on-Chip (NoC)

So:

Crossbar ⊂ Interconnect

Key Characteristics

✅ Scales better
✅ Lower area possible
❌ Slightly more latency
❌ More complex internally

When Interconnect is Used

- Medium to large SoCs

- Many masters/slaves

- Multiple clock domains

- Power & security domains

Example:

- CPU clusters

- GPU

- PCIe

- Video blocks

Verification Focus

- Address map correctness

- Clock domain crossing

- QoS / priority

- Deadlock freedom

- Security rules

📌 PCIe analogy:
AXI interconnect ≈ PCIe fabric (switch + bridges + routing)

🔹 Direct Comparison Table

| Feature | AXI Crossbar | AXI Interconnect |
| --- | --- | --- |
| Definition | Switching fabric | Complete connectivity solution |
| Parallel paths | Full | Depends on design |
| Scalability | Limited | High |
| Area | High | Optimized |
| Latency | Low | Medium |
| Complexity | Low–medium | High |
| Used in | Small SoCs | Large SoCs |
| Includes | Only switch | Switch + logic |

🔹 Real SoC Example

Small SoC

- CPU

- DMA

- DDR

- UART

👉 AXI crossbar is enough

Large SoC (Mobile / Automotive)

- Multi-core CPU

- GPU

- ISP

- PCIe

- Display

👉 AXI interconnect / NoC required

🔹 Verification Reality (Important)

In real projects:

- You rarely verify raw crossbar RTL

- You verify system behavior through the interconnect

Typical tests:

- Multiple masters hammer DDR

- Starvation checks

- Backpressure

- Reset during traffic

🔹 Interview Trap ⚠️

❌ “Crossbar and interconnect are same”
✅ “Crossbar is a form of interconnect optimized for parallel access”

🔹 One-Line Summary

AXI crossbar is a specific high-performance switching structure, while AXI interconnect is the complete AXI connectivity solution that may contain crossbars, buses, bridges, and NoC components.

If you want next, I can explain:

- AXI NoC vs crossbar

- AXI arbitration algorithms

- Typical AXI interconnect bugs

- UVM testcases for interconnect

Just say 👍

Below is a clear, verification-engineer-oriented explanation of how arbitration works internally in an SoC, with AXI/NoC context, corner cases, and what to verify (this is exactly how interviewers expect you to explain it).

How Arbitration Works Internally (SoC Perspective)

Arbitration decides which master gets access to a shared resource (bus, memory, slave, or NoC port) when multiple requests arrive simultaneously.

1. Where Arbitration Exists in an SoC

Arbitration is present at multiple levels:

- Bus / Interconnect arbitration

  - AXI interconnect (multiple masters → single slave)

- Memory controller arbitration

  - CPU, DMA, PCIe competing for DRAM

- NoC router arbitration

  - Multiple input ports → one output port

- Peripheral arbitration

  - Multiple DMA channels → single peripheral

2. Internal Arbitration Flow (Step-by-Step)

Step 1: Request Generation

Each master asserts a request:

- AXI: ARVALID / AWVALID

- NoC: Flit request

- Memory controller: Read/write request

Each request carries attributes:

- ID

- Priority

- QoS

- Type (read/write)

- Burst length

Step 2: Request Queueing

Requests are stored in:

- FIFOs

- Request queues

- Input buffers

Why?

- To decouple timing

- To allow reordering

- To support outstanding transactions

Step 3: Arbitration Logic

Arbiter examines all valid requests and selects one winner per cycle (or per arbitration window).

Common Arbitration Algorithms:

1️⃣ Fixed Priority

- Highest priority always wins

Example:

Master0 > Master1 > Master2

✔ Simple
❌ Starvation risk

2️⃣ Round-Robin

- Rotates priority among requesters

Example:

Last granted = M1 → Next priority starts at M2

✔ Fair
❌ Slightly more logic

3️⃣ Weighted Round-Robin

- Each master gets weight-based bandwidth

Example:

CPU: 4 slots

DMA: 2 slots

PCIe: 1 slot

✔ QoS control
❌ Complex verification

4️⃣ QoS-Based Arbitration (AXI / NoC)

- Uses AWQOS / ARQOS

Decision based on:

Highest QoS → Fairness → Aging

3. Grant Phase

- Arbiter asserts grant signal to winning master

- Grant may last:

  - 1 beat

  - Entire burst

  - Multiple flits (NoC)

Important detail:

- Some arbiters are beat-level

- Others are transaction-level

4. Data Transfer Phase

- Winner drives address/data

- Losers remain pending

- Backpressure may pause transfer

AXI example:

AWVALID → AWREADY

WVALID  → WREADY

5. Release & Next Arbitration

- Grant released when:

  - Burst completes

  - Target stalls

  - Preemption occurs (if supported)

- Arbiter updates internal pointer/state

- Next arbitration cycle begins

6. Special Internal Features

A. Burst Locking

- Arbiter locks resource for entire burst

- Prevents interleaving

- Important for atomic operations

B. Reordering Support

- Requests may complete out of order

- IDs track responses

- Arbiter ensures ordering rules are respected

C. Starvation Prevention

- Aging counters

- Priority boost after wait threshold

D. Deadlock Avoidance

- Separate read/write paths

- Credit-based flow control

- Independent arbitration domains

7. Example: AXI Interconnect Arbitration

Scenario:
CPU, DMA, PCIe all issue reads to DDR.

Internal steps:

- All assert ARVALID

- Interconnect queues requests

- Arbiter evaluates:

  - ARQOS

  - Round-robin pointer

- Grants one master

- Forwards address to DDR

- Tracks outstanding ID

- Repeats

8. What Verification Engineers Must Check

Functional Checks

✔ Only one grant per resource
✔ Correct master wins as per policy
✔ No transaction loss

Fairness & Starvation

✔ Low-priority master eventually gets access
✔ Aging logic works

Protocol Compliance

✔ AXI ordering rules
✔ Burst integrity
✔ Atomicity

Stress Scenarios

✔ All masters active
✔ Backpressure from slave
✔ Long bursts blocking short ones

9. Typical Arbitration Bugs

🚫 Starvation of low-priority master
🚫 Grant stuck due to reset issue
🚫 Burst cut mid-way
🚫 QoS ignored
🚫 Deadlock under heavy load

10. Interview-Ready One-Line Summary

Arbitration internally collects concurrent requests, applies a policy (priority/round-robin/QoS), grants exclusive access to one requester while tracking outstanding transactions, and ensures fairness, ordering, and deadlock-free operation.

If you want, I can also:

- 🔹 Explain AXI arbitration with timing diagram

- 🔹 Show UVM testcases to verify arbitration

- 🔹 Cover NoC router arbitration

- 🔹 Provide interview Q&A on arbitration

Just tell me 👍

Below are practical, interview-ready UVM testcases to verify arbitration, written from a real SoC / AXI-interconnect verification point of view.
I’ll show what to test, how to structure it, and sample UVM code (simplified but realistic).

UVM Testcases to Verify Arbitration

We assume:

- Multiple masters → single shared slave (AXI/NoC/Memory)

- Arbitration policy: Round-Robin / Priority / QoS

- Central interconnect arbiter

1️⃣ Basic Arbitration Test (Multiple Masters Competing)

Goal

Verify:
✔ Only one master granted at a time
✔ No transaction loss

Test Idea

- All masters issue requests at the same time

- Check:

  - Requests complete

  - No overlapping grants

Virtual Sequence

class arb_basic_vseq extends uvm_sequence;

`uvm_object_utils(arb_basic_vseq)

virtual task body();

fork

mseqr.cpu_seq.start(cpu_read_seq::type_id::create("cpu_seq"));

mseqr.dma_seq.start(dma_read_seq::type_id::create("dma_seq"));

mseqr.pcie_seq.start(pcie_read_seq::type_id::create("pcie_seq"));

join

endtask

endclass

Scoreboard Check

assert ($onehot0(grant_vector))

else `uvm_error("ARB", "Multiple grants detected!");

2️⃣ Fixed Priority Arbitration Test

Goal

Verify:
✔ High-priority master always wins
✔ Low-priority master waits

Test Scenario

Priority:

CPU > DMA > PCIe

Sequence

class fixed_prio_vseq extends arb_basic_vseq;

virtual task body();

cpu_seq.priority  = 3;

dma_seq.priority  = 2;

pcie_seq.priority = 1;

super.body();

endtask

endclass

Expected Behavior

- CPU completes first

- DMA next

- PCIe last

Scoreboard Check

if (pcie_txn.completed_before(cpu_txn))

`uvm_error("ARB", "Priority violation detected");

3️⃣ Round-Robin Fairness Test (No Starvation)

Goal

Verify:
✔ Every master eventually gets service
✔ No starvation

Test Idea

- Continuous traffic from all masters

- Track grant order

Grant Tracking

int grant_count[string];

function void record_grant(string master);

grant_count[master]++;

endfunction

End-of-Test Check

foreach (grant_count[m]) begin

if (grant_count[m] == 0)

`uvm_error("ARB", $sformatf("Starvation detected for %s", m));

end

4️⃣ QoS-Based Arbitration Test (AXI)

Goal

Verify:
✔ ARQOS/AWQOS affects arbitration

Sequence Example

class qos_vseq extends arb_basic_vseq;

virtual task body();

cpu_seq.qos  = 4;

dma_seq.qos  = 1;

pcie_seq.qos = 0;

super.body();

endtask

endclass

Expected Result

- CPU gets higher bandwidth

- DMA & PCIe still make progress

Coverage

covergroup qos_cg;

coverpoint qos_value;

coverpoint grant_master;

cross qos_value, grant_master;

endgroup

5️⃣ Backpressure Arbitration Test

Goal

Verify:
✔ Arbiter handles slave stall
✔ Pending masters are not dropped

Test Scenario

- Slave deasserts READY

- Multiple masters waiting

Monitor Check

if (slave_ready == 0 && grant_changes)

`uvm_error("ARB", "Grant changed during backpressure");

6️⃣ Burst Locking Test

Goal

Verify:
✔ Grant is held for entire burst
✔ No mid-burst switching

Sequence

cpu_seq.burst_len = 16;

dma_seq.burst_len = 1;

Assertion

assert property (@(posedge clk)

grant_cpu |-> grant_cpu throughout burst_active

);

7️⃣ Stress Arbitration Test (Real Project Test)

Goal

Verify:
✔ Stability under heavy load

Test Characteristics

- Random masters

- Random burst length

- Random QoS

- Long simulation

Randomized Sequence

class arb_stress_vseq extends uvm_sequence;

virtual task body();

repeat (10000) begin

automatic int sel = $urandom_range(0,2);

case (sel)

0: cpu_seq.start(mseqr.cpu_seq);

1: dma_seq.start(mseqr.dma_seq);

2: pcie_seq.start(mseqr.pcie_seq);

endcase

end

endtask

endclass

8️⃣ Common Arbitration Assertions (Must-Have)

// Only one grant active

assert property (@(posedge clk)

$onehot0(grant_vector)

);

// Grant only if request exists

assert property (@(posedge clk)

grant_master |-> req_master

);

9️⃣ Interview-Ready Summary

I verify arbitration by generating simultaneous traffic from multiple masters, checking grant exclusivity, fairness, priority/QoS behavior, burst locking, and robustness under backpressure and stress using virtual sequences, scoreboards, assertions, and coverage.

If you want next

I can:

- 🔹 Map this to AXI signals (AR/AW/W/R/B)

- 🔹 Show NoC router arbitration tests

- 🔹 Provide full UVM mini-project for arbiter

- 🔹 Give interview Q&A with expected answers

Just tell me 👍

* SoC Integration

Got it 👍
This “SoC Integration” picture is summarizing how everything is connected and works together inside an SoC.
Let me explain it in simple words, tying it back to what you already understood about CPU, DMA, interrupts.

What “SoC Integration” means

SoC integration means:

Putting CPU + peripherals + interconnect + interrupt controller together so they work as one system.

Typical blocks in SoC integration

1️⃣ CPU (Processor)

- Runs software

- Programs peripherals using bus writes/reads

- Handles interrupts

2️⃣ Interconnect (AXI / AHB / APB / NoC)

- Connects CPU to peripherals

- Handles:

  - Arbitration

  - QoS

  - Address decoding

CPU ⇄ Interconnect ⇄ Peripherals

3️⃣ Peripherals (UART, DMA, Timer, GPIO, SPI, etc.)

- Do hardware work

- Have registers

- May generate interrupts

Examples:

- UART → RX/TX events

- DMA → transfer complete

- Timer → timeout

4️⃣ Interrupt Controller (INTC / GIC / PLIC)

- Collects interrupt signals from peripherals

- Decides:

  - Priority

  - Masking

  - Which CPU/core gets the interrupt

Peripherals → INTC → CPU

Putting it all together (full flow)

Example: DMA in SoC Integration

- CPU programs DMA registers (via AXI/APB)

- CPU enables DMA interrupt

- CPU starts DMA

- DMA transfers data as bus master

- DMA finishes

- DMA raises interrupt

- INTC forwards interrupt to CPU

- CPU ISR runs

- CPU continues previous work

This is exactly what SoC integration enables.

Key idea (very important)

- CPU controls → via bus

- Peripherals notify → via interrupt

- INTC manages interrupts

- Interconnect manages data movement

One-line summary (remember this)

SoC integration connects CPU, peripherals, interconnect, and interrupt controller so the CPU can control hardware and hardware can notify the CPU using interrupts.

You now have a complete and correct SoC mental model 👌
If you want next, I can explain:

- SoC integration from verification point of view

- Interrupt integration checklist

- AXI/APB + interrupt timing

- Real interview explanation for “Explain SoC integration”

Just tell me what you want next 🚀

SOC VS IP VS Sub_System = integration

Nice — you’re thinking in the right direction, just need to clean up the concept 👍
Let me restate it very clearly and then fix the confusion.

Short answer first ✅

YES, in real SoCs:

IPs are usually grouped into subsystems first, and then subsystems are connected together at SoC level.

So your thinking is correct ✔️

Correct mental model (very important) 🧠

There are 3 levels, not just one:

1️⃣ IP level

2️⃣ Subsystem level

3️⃣ SoC (Top) level

1️⃣ IP Level (lowest level)

Single functional blocks:

- UART

- SPI

- GPIO

- Timer

- DMA

- DDR controller

Each IP:

- Has its own registers

- Has its own protocol (AXI/APB)

- Can work alone in IP-level verification

Example:

UART IP

DMA IP

2️⃣ Subsystem Level (middle level) 🔗

A subsystem = group of related IPs connected together.

Example: Peripheral Subsystem

Contains:

- UART

- SPI

- I2C

- GPIO

- Timers

- APB interconnect

- Clock gating

- Local reset

CPU

|

AXI

|

APB Bridge

|

Peripheral Subsystem

├── UART

├── SPI

├── GPIO

└── Timer

Here:

- IPs talk inside the subsystem

- Subsystem exposes one clean interface to SoC

3️⃣ SoC Level (top level) 🏗️

Now we connect subsystems, not every IP directly.

Typical SoC:

- CPU Subsystem

- Memory Subsystem

- Peripheral Subsystem

- Debug Subsystem

- Power Management Subsystem

CPU Subsystem

|

AXI / NoC

/          \

Memory Subsystem   Peripheral Subsystem

This is SoC integration

Key clarification (your confusion point) 🔑

You said:

“i thorugh ip are connect in susbsystem”

Corrected sentence:

IPs are connected inside subsystems, and subsystems are connected together in the SoC.

Not wrong — just incomplete earlier.

Why do we use subsystems? 🤔

1️⃣ Scalability

- 100+ IPs → impossible to manage flat

- Subsystems simplify integration

2️⃣ Reuse

- Same peripheral subsystem reused in many SoCs

3️⃣ Cleaner verification

- Verify subsystem once

- Reuse at SoC level

4️⃣ Clock / Reset control

- Each subsystem can have its own clock & reset domain

Real example: UART access (full flow)

CPU

→ AXI

→ AXI-to-APB Bridge

→ Peripheral Subsystem

→ UART

From CPU point of view:

- It talks to SoC address

- Doesn’t care UART is deep inside subsystem

One-line final definition 🔥

SoC Integration = connecting multiple subsystems (each made of IPs) and verifying they work together as one complete chip.

If you want next:

- Difference between IP vs Subsystem vs SoC

- Real SoC block diagram explanation

- Integration verification checklist (industry style)

Just tell me 👍

EXAMPLE_of_Integration

Perfect 👍
Let’s do this very practically:

1️⃣ Draw SoC integration diagram
2️⃣ Show simple RTL code that integrates a subsystem into an SoC
3️⃣ Explain what part is “integration” (this is the key)

I’ll keep it simple, realistic, and interview-ready.

1️⃣ SoC Integration – Block Diagram (Text Drawing)

Example: CPU + Interconnect + UART + DMA + Memory

┌────────────┐

│    CPU     │

│ (AXI M)    │

└─────┬──────┘

│

┌─────▼──────┐

│ AXI        │

│ Interconnect│

└─────┬──────┘

┌───────────┼───────────┐

│           │           │

┌─────▼───┐ ┌─────▼───┐ ┌─────▼────┐

│  DDR    │ │  DMA    │ │ AXI→APB  │

│ Memory  │ │ (AXI M) │ │  Bridge  │

└─────────┘ └─────┬───┘ └─────┬────┘

│           │

│      ┌────▼────┐

│      │  UART   │

│      │ (APB S) │

│      └────┬────┘

│           │

└────────► Interrupt

│

┌──────▼──────┐

│ Interrupt   │

│ Controller  │

└──────┬──────┘

│

CPU

👉 This entire wiring is SoC integration

2️⃣ Simple RTL Example – SoC Integration Code

We’ll integrate:

- CPU (dummy)

- AXI Interconnect (dummy)

- UART (APB)

- Interrupt wiring

⚠️ This is structural integration code, not IP internals.

🔹 UART APB Slave (simplified)

module uart_apb (

input        PCLK,

input        PRESETn,

input        PSEL,

input        PENABLE,

input        PWRITE,

input [31:0] PADDR,

input [31:0] PWDATA,

output reg [31:0] PRDATA,

output reg   PREADY,

output reg   uart_irq

);

reg [7:0] tx_reg;

reg [7:0] rx_reg;

always @(posedge PCLK or negedge PRESETn) begin

if (!PRESETn) begin

tx_reg   <= 0;

rx_reg   <= 0;

uart_irq <= 0;

end else if (PSEL && PENABLE && PWRITE) begin

tx_reg <= PWDATA[7:0];   // write TX

end

end

always @(*) begin

PRDATA = {24'd0, rx_reg};

PREADY = 1'b1;

end

endmodule

🔹 Dummy CPU (AXI Master – abstracted)

module cpu (

input clk,

input rstn,

output [31:0] addr,

output        write,

output [31:0] wdata,

input         irq

);

assign addr  = 32'h4000_0000; // UART base address

assign write = 1'b1;

assign wdata = 32'h55;

endmodule

🔹 SoC Top Module (THIS IS INTEGRATION)

module soc_top (

input clk,

input rstn

);

// CPU signals

wire [31:0] cpu_addr;

wire        cpu_write;

wire [31:0] cpu_wdata;

wire        uart_irq;

// APB signals

wire        PSEL, PENABLE, PWRITE;

wire [31:0] PADDR, PWDATA, PRDATA;

// -----------------------------

// CPU instance

// -----------------------------

cpu u_cpu (

.clk   (clk),

.rstn  (rstn),

.addr  (cpu_addr),

.write (cpu_write),

.wdata (cpu_wdata),

.irq   (uart_irq)

);

// -----------------------------

// Address decode (INTEGRATION LOGIC)

// -----------------------------

assign PSEL    = (cpu_addr == 32'h4000_0000);

assign PENABLE = PSEL;

assign PWRITE  = cpu_write;

assign PADDR   = cpu_addr;

assign PWDATA  = cpu_wdata;

// -----------------------------

// UART instance

// -----------------------------

uart_apb u_uart (

.PCLK     (clk),

.PRESETn  (rstn),

.PSEL     (PSEL),

.PENABLE  (PENABLE),

.PWRITE   (PWRITE),

.PADDR    (PADDR),

.PWDATA   (PWDATA),

.PRDATA   (PRDATA),

.PREADY   (),

.uart_irq (uart_irq)

);

endmodule

3️⃣ What Part Is SoC Integration Here?

✅ Integration is:

- Address mapping (0x4000_0000 → UART)

- Bus connections

- Interrupt wiring

- Clock & reset hookup

❌ Integration is NOT:

- UART internal logic

- CPU internal logic

4️⃣ Subsystem Integration vs SoC Integration

UART Subsystem

UART + APB + Interrupt

SoC Integration

CPU + DMA + Memory + UART Subsystem + Interrupt Controller

Subsystem = small group
SoC = all subsystems together

5️⃣ How Verification Thinks About This 🧠

As an SoC verification engineer, you test:

- CPU writes UART register → does UART work?

- UART interrupt → does CPU see it?

- Two masters → arbitration works?

- Reset during transaction → safe or not?

🔑 One-Line Summary

SoC integration code is mostly wiring, address mapping, clocks, resets, and interrupts — not logic.

If you want next:

- UVM SoC integration testbench

- AXI/APB real integration errors

- Interview questions on SoC integration

Just tell me 👍

EXAMPLE_CODE_FOR_SOC_INTEGRATION

Perfect 👍
Here is ONE clean, minimal SoC integration example — exactly at the level a verification engineer must understand.

No UVM, no noise.
Just pure SoC integration RTL.

✅ Example: Simple SoC Integration Code

CPU + APB Interconnect + UART + Interrupt

1️⃣ What this SoC contains

- CPU (master)

- UART (peripheral)

- APB fabric (very simple)

- Interrupt wiring

This is what SoC integration really looks like.

2️⃣ UART IP (already designed, not integration)

module uart (

input        clk,

input        rstn,

input        psel,

input        penable,

input        pwrite,

input [31:0] paddr,

input [31:0] pwdata,

output reg   irq

);

always @(posedge clk or negedge rstn) begin

if (!rstn)

irq <= 1'b0;

else if (psel && penable && pwrite)

irq <= 1'b1;   // raise interrupt on write

else

irq <= 1'b0;

end

endmodule

3️⃣ CPU (very abstract)

module cpu (

output reg [31:0] addr,

output reg        write,

output reg [31:0] wdata,

input             irq

);

initial begin

addr  = 32'h4000_0000; // UART base

write = 1'b1;

wdata = 32'h55;

end

endmodule

4️⃣ ⭐ SoC TOP — THIS IS INTEGRATION CODE

module soc_top (

input clk,

input rstn

);

// CPU signals

wire [31:0] cpu_addr;

wire        cpu_write;

wire [31:0] cpu_wdata;

wire        uart_irq;

// APB signals

wire psel_uart;

wire penable;

// -------------------------

// CPU instance

// -------------------------

cpu u_cpu (

.addr  (cpu_addr),

.write (cpu_write),

.wdata (cpu_wdata),

.irq   (uart_irq)

);

// -------------------------

// Address decode (INTEGRATION)

// UART @ 0x4000_0000

// -------------------------

assign psel_uart = (cpu_addr == 32'h4000_0000);

assign penable   = psel_uart;

// -------------------------

// UART instance

// -------------------------

uart u_uart (

.clk     (clk),

.rstn    (rstn),

.psel    (psel_uart),

.penable (penable),

.pwrite  (cpu_write),

.paddr   (cpu_addr),

.pwdata  (cpu_wdata),

.irq     (uart_irq)

);

endmodule

5️⃣ Why THIS is SoC integration (important)

✅ This code shows:

- IP instantiation

- Address decoding

- Bus connection

- Interrupt wiring

- Clock/reset hookup

❌ This code does NOT show:

- UART internals

- CPU internals

- Protocol details

That’s exactly what SoC integration is.

6️⃣ Diagram for this code 🧠

CPU

│  addr/write/data

▼

Address Decode

│

▼

UART  ────► IRQ ────► CPU

🔑 One-line takeaway (memorize)

SoC integration code is mostly wiring, address mapping, and interrupt connections — not logic.

If you want next:

- Same example with AXI

- Same example with DMA

- Same example from DV debugging point of view

Just tell me 👍

// soc code:-

Yes 👍 — that’s exactly the idea.
Let me lock it in clearly so there’s zero confusion.

✅ What this line really means

assign psel_uart = (cpu_addr == 32'h4000_0000);

This means:

If CPU accesses address 0x4000_0000, select the UART subsystem

So yes:

- UART has one address range

- Other addresses go to other subsystems

This is called address decoding.

🔁 Real SoC view (not just UART)

In a real SoC, CPU does this:

CPU address = X

|

v

Address Decoder / Interconnect

|

+-- UART subsystem

+-- GPIO subsystem

+-- TIMER subsystem

+-- DMA subsystem

CPU does NOT know which subsystem
It just puts an address on the bus.

🧠 Better (more realistic) decode example

Instead of single address, we decode a range:

// UART: 0x4000_0000 – 0x4000_00FF

assign psel_uart = (cpu_addr[31:8] == 24'h400000);

// GPIO: 0x4000_0100 – 0x4000_01FF

assign psel_gpio = (cpu_addr[31:8] == 24'h400001);

// TIMER: 0x4000_0200 – 0x4000_02FF

assign psel_timer = (cpu_addr[31:8] == 24'h400002);

Now:

- Same CPU

- Same bus

- Different subsystems based on address

❗ Important correction (DV interview trap)

❌ CPU does NOT define psel_uart
✅ Interconnect / address decoder defines it

CPU only outputs:

addr, write, wdata

The SoC fabric decides where it goes.

🔍 From verification engineer POV

You verify:

- Correct address → correct subsystem

- Wrong address → no subsystem selected

- No overlap between address ranges

- No two psel_* high together

Typical DV assertion:

assert ($onehot0({psel_uart, psel_gpio, psel_timer}));

🧩 Visual mental model

CPU puts address

|

v

Interconnect decodes

|

+-- UART (0x4000_0000)

+-- GPIO (0x4000_0100)

+-- TIMER (0x4000_0200)

🔑 One-line summary (memorize)

CPU only generates an address; the SoC interconnect uses that address to select the correct subsystem.

You’ve got the concept right ✅
If you want, next I can show:

- Address decode bugs DV catches

- AXI version of this

- How software sees this (driver view)

Just tell me 👍

QUEstion:- address assign for each sub system inside interconnect and cpu have addrreess varaible generate addresss cpu don't have idea about which generate that address for which sub system

Ans:-

YES ✅ — you’ve got it exactly right.
Let me restate it cleanly and then lock it in with a tiny example so it sticks forever.

✅ Correct mental model (this is the truth)

CPU only generates an address.
Interconnect decides which subsystem gets selected.
CPU has NO idea which subsystem is behind that address.

That statement is 100% correct.

🔹 Who does what (very important)

CPU

- Generates:

  - addr

  - read/write

  - data

- Based on:

  - software instruction

- ❌ Does NOT know:

  - UART / GPIO / DMA

  - Interconnect

  - Address decode logic

CPU just says:

“Access address 0x4000_0000”

Interconnect / Address Decoder

- Looks at addr

- Decides:

  - Which subsystem

  - Which slave

- Generates:

  - psel_uart

  - psel_gpio

  - psel_dma

This is where address assignment lives.

🔹 Why CPU truly doesn’t know

From software view:

*(volatile int*)0x40000000 = 0x55;

CPU executes:

- Load address into register

- Issue bus transaction

CPU doesn’t know:

- That this is UART

- That this is APB

- That there’s an interrupt

All knowledge is in:

- Memory map

- Interconnect decode

🔹 Tiny RTL example (perfect summary)

CPU (address only)

assign cpu_addr = addr_from_instruction;

Interconnect (decoding)

assign sel_uart = (cpu_addr[31:12] == 20'h40000);

assign sel_gpio = (cpu_addr[31:12] == 20'h40001);

assign sel_dma  = (cpu_addr[31:12] == 20'h40002);

CPU is not involved here.

🔹 Visual picture (lock this in)

Software writes 0x4000_0000

|

v

CPU

(addr = 0x4000_0000)

|

v

Interconnect Decode

|        |       |

UART     GPIO     DMA

🔹 From verification engineer POV (this is gold)

You verify:

- Address map correctness

- No overlap

- Correct routing

- Default slave on invalid address

Typical DV check:

assert ($onehot0({sel_uart, sel_gpio, sel_dma}));

🔑 One-line interview answer (memorize)

“The CPU only generates addresses based on software. The SoC interconnect decodes those addresses and routes transactions to the correct subsystem.”

You are thinking exactly like a real SoC DV engineer now 👌
If you want next:

- Address-map bugs seen in real chips

- AXI interconnect version

- How DV tests wrong/illegal addresses

Just tell me 👍

Question:- where integration inside cpu ya inside interconnect(axi/crossbar/noc)?

Ans:-

Short, clear answer first 👇

✅ Integration is inside the INTERCONNECT / SoC TOP — NOT inside the CPU

Now let’s make it crystal clear so there’s no confusion later.

🔹 Where integration does NOT happen

❌ Inside CPU

CPU:

- Generates address

- Generates read/write

- Executes instructions

CPU does NOT:

- Know UART / GPIO / DMA

- Do address decoding

- Select subsystems

- Wire interrupts internally

CPU is a generic master.

🔹 Where integration ACTUALLY happens

✅ Inside Interconnect + SoC top

This includes:

- AXI/APB interconnect

- Address decoder

- Bus bridges

- Interrupt wiring

- Clock/reset distribution

This is called:

- SoC top

- Fabric

- Integration layer

🔹 Visual (very important)

Software

|

v

CPU (addr, rw, data)

|

v

--------------------------------

|  Interconnect / Fabric       |

|  - Address decode            |

|  - Arbitration               |

|  - Routing                   |

--------------------------------

|        |        |

UART     GPIO     DMA

CPU is outside integration logic.

🔹 RTL proof (tiny example)

CPU (no integration)

assign cpu_addr = instr_addr;

Interconnect (integration)

assign sel_uart = (cpu_addr[31:8] == 24'h400000);

assign sel_gpio = (cpu_addr[31:8] == 24'h400001);

This logic never lives in CPU.

🔹 Why this separation exists (real reason)

- CPU IP is reused across many SoCs

- Different SoCs have different address maps

- CPU must stay generic

So:

Address map belongs to SoC, not CPU

🔹 From verification engineer POV

You mostly debug:

- Interconnect RTL

- Address decode

- Glue logic

Very rarely CPU RTL.

🔑 One-line final answer (interview-ready)

SoC integration is implemented in the interconnect and SoC top level, while the CPU only generates addresses and is unaware of subsystem selection.

You’re asking exactly the right questions — this is core SoC understanding 👍
If you want, next I can show:

- What integration logic files look like in real projects

- Common integration bugs

- How DV debugs interconnect issues step-by-step

Question:- soc integration happpend inside soc_top_file and interconnect(axi/cross_bar/noc)?

Ans:- yes, that’s exactly correct.

Yes ✅ — that’s exactly correct.
Let me say it cleanly and then break it down so it’s locked in forever.

✅ Final Correct Statement

SoC integration happens in the soc_top file and in the interconnect (AXI crossbar / NoC).

CPU and peripherals are plugged into this integration — they do not do it.

🔹 Where integration happens (exact places)

1️⃣ soc_top (Top-level integration)

This is the root integration file.

It handles:

- Instantiating CPUs

- Instantiating interconnects

- Instantiating subsystems

- Clock & reset hookup

- Interrupt wiring between blocks

- Power domain connections

Think of soc_top as the motherboard.

2️⃣ Interconnect (AXI / Crossbar / NoC)

This is where most real integration logic lives.

It handles:

- Address decoding

- Routing transactions

- Arbitration between masters

- QoS & ordering

- Back-pressure handling

Examples:

- AXI crossbar

- AXI interconnect IP

- Network-on-Chip (NoC)

🔹 Where integration does NOT happen

❌ Inside CPU

- CPU only generates addresses

- No knowledge of SoC map

❌ Inside leaf IPs (UART, DMA, etc.)

- They respond when selected

- No knowledge of full system

🔹 Real SoC structure (mental picture)

soc_top

┌──────────────────────────────────┐

│                                  │

│   CPU(s)                          │

│      │                            │

│      ▼                            │

│  AXI Interconnect / NoC           │

│      │      │      │              │

│      ▼      ▼      ▼              │

│  DMA     UART_SS  GPIO_SS          │

│                                  │

│  Interrupt Controller (GIC)       │

│  Clock & Reset Manager            │

└──────────────────────────────────┘

🔹 How this maps to files (real projects)

| File | Role |
| --- | --- |
| cpu_top.v | CPU IP |
| axi_xbar.v | Address decode + routing |
| uart_subsystem.v | Peripheral subsystem |
| gic_top.v | Interrupt integration |
| soc_top.v | All system integration |

🔹 From verification engineer POV (important)

You mostly debug:

- soc_top.v

- Interconnect configs

- Address maps

- Interrupt routing tables

Not CPU internals.

🔑 One-line interview answer (perfect)

SoC integration is implemented in the SoC top-level module and the interconnect fabric, while CPUs and peripherals remain generic and unaware of system-level connections.

You’re thinking in exact SoC architect + DV mindset now 👌
If you want next:

- A real SoC directory structure

- Integration bug examples

- How DV verifies interconnect configs

Question:- cross_bar   vs noc vs axi_interconnect difference?

Ans:-

Great question — this is classic SoC interview + real-project knowledge 👍
I’ll explain it simply first, then go deeper, and finally give you a when-to-use-what table.

1️⃣ One-line intuition (easy to remember)

- AXI Interconnect → standard ARM-style fabric for small–medium SoCs

- Crossbar → simple, fast, centralized switch

- NoC → scalable, packet-based network for large SoCs

2️⃣ What problem they all solve

All three do the same basic job:

Connect multiple masters (CPU, DMA) to multiple slaves (DDR, peripherals)

Difference is how they scale and how complex they are.

3️⃣ AXI Interconnect

What it is

- ARM-defined interconnect IP

- Uses AXI protocol

- Centralized logic

Characteristics

- Address decode

- Arbitration

- Supports AXI features (ID, bursts, QoS)

- Often configurable (Vivado, Synopsys, etc.)

Typical use

- Small to medium SoCs

- 1–4 CPUs, few DMAs

- Many peripherals

Mental picture

CPU ──┐

DMA ──┼── AXI Interconnect ── DDR

GPU ──┘

Pros

✅ Simple
✅ AXI compliant
✅ Easy DV

Cons

❌ Limited scalability
❌ Central bottleneck

4️⃣ Crossbar

What it is

- Pure switch matrix

- Direct paths between masters and slaves

Characteristics

- Multiple simultaneous connections

- Combinational or pipelined

- Can be AXI, AHB, or custom

Mental picture

┌── DDR

CPU ───┼── UART

DMA ───┼── GPIO

GPU ───┘

Pros

✅ High performance
✅ Low latency

Cons

❌ Area explodes with size
❌ Hard to scale beyond few ports

Typical use

- Small high-performance blocks

- L2 cache ↔ masters

- Subsystems

5️⃣ NoC (Network-on-Chip)

What it is

- Packet-based on-chip network

- Routers + links

- Similar to Ethernet but on silicon

Characteristics

- Distributed routing

- Virtual channels

- QoS, deadlock avoidance

- Power & clock domain aware

Mental picture

CPU ──┐      ┌── DDR

├── Router ── Router ── GPU

DMA ──┘      └── NPU

Pros

✅ Extremely scalable
✅ High bandwidth
✅ Handles many masters

Cons

❌ Complex
❌ DV is hard
❌ Latency higher than crossbar

Typical use

- Mobile SoCs

- AI accelerators

- Multi-die systems

6️⃣ Side-by-side comparison (very important)

| Feature | AXI Interconnect | Crossbar | NoC |
| --- | --- | --- | --- |
| Architecture | Centralized | Central switch | Distributed |
| Scalability | Medium | Low | Very high |
| Latency | Medium | Low | Medium–High |
| Area | Medium | High (large) | Optimized |
| Complexity | Low–Medium | Low | High |
| Protocol | AXI | Any | Packet-based |
| DV Effort | Medium | Low | High |

7️⃣ Real SoC usage (truth from industry)

- UART/GPIO side → AXI Interconnect

- CPU ↔ L2 cache → Crossbar

- CPU/GPU/AI ↔ DDR → NoC

Most modern SoCs use ALL THREE together.

8️⃣ Verification engineer perspective 👀

What DV focuses on

AXI Interconnect

- Address decode

- Ordering

- QoS

- Deadlock

Crossbar

- Arbitration

- Simultaneous access

- Starvation

NoC

- Packet routing

- Congestion

- VC deadlock

- Power/clock crossing

9️⃣ Interview-ready one-liner (gold)

“AXI interconnects and crossbars are centralized fabrics suitable for small to medium SoCs, while NoCs use distributed routing to scale efficiently for large, high-bandwidth systems.”

If you want next:

- Real integration bugs in each

- How DV testbenches differ

- Simple RTL example comparing them

Just tell me 👍

Question:- arbitration and arbiter are same think?

Ans:-

Arbitration and arbiter are dependent, but conceptually different.

- Arbitration cannot exist without an arbiter

- An arbiter is useless without arbitration logic

- “Arbitration is implemented inside the arbiter”

Short answer: they’re related, but not exactly the same thing 🙂

🧠 Simple way to remember

- Arbitration → the process / policy (what decision is made)

- Arbiter → the hardware/logic block (who makes that decision)

👉 Think:

Arbitration = rule
Arbiter = referee

🏟️ Real-world analogy

- Arbitration:
“Only one person can speak at a time. Priority goes to seniors. If equal, round-robin.”

- Arbiter:
The actual person who enforces those rules and says:
👉 “You speak now.”

🧩 In SoC / interconnect terms

Arbitration

- The algorithm / scheme

- Examples:

  - Fixed priority

  - Round-robin

  - Weighted round-robin

  - QoS-based

Arbiter

- The RTL block that:

  - Looks at request signals

  - Applies the arbitration rule

  - Generates grant signals

🧪 Example (AXI / Crossbar)

req_cpu  ──┐

req_dma  ──┼──>  ARBITER  ──> grant_cpu / grant_dma

req_gpu  ──┘

- Arbiter = this block

- Arbitration = logic inside this block deciding who wins

🛠️ Verification angle (important)

You verify arbitration behavior, not just the arbiter RTL.

Example checks:

- Is round-robin truly fair?

- Does low-priority master starve?

- Are QoS rules respected?

So in DV language:

“We verify the arbitration logic implemented inside the arbiter.”

📌 One-line interview answer

“Arbitration is the decision-making policy, while an arbiter is the hardware block that implements that policy.”

If you want, I can:

- Show RTL of a simple arbiter

- Explain round-robin vs priority with timing

- Show real SoC arbitration bugs

Just say the word 👌

Question:- how many step for soc integration?

Ans:-

Great question 👍 — SoC integration looks messy from outside, but in reality it follows a clear sequence.
I’ll give you a clean, verification-engineer-friendly view.

✅ High-level answer

SoC integration typically happens in ~10 logical steps
(Some teams say 8, some say 12 — but the flow is the same.)

🧩 Step-by-step SoC integration flow (practical)

1️⃣ Architecture & block identification

Decide what goes into the SoC:

- CPU / CPUs

- Interconnect (AXI, NoC)

- Subsystems (DMA, UART, PCIe, etc.)

- Interrupt controller

- Clock/reset/power blocks

📌 DV relevance: Know all blocks and interfaces.

2️⃣ Address map definition

Assign address ranges to each subsystem:

UART   → 0x4000_0000 – 0x4000_0FFF

DMA    → 0x4001_0000 – 0x4001_0FFF

GPIO   → 0x4002_0000 – 0x4002_0FFF

📌 CPU does NOT know this mapping — interconnect does.

3️⃣ Interconnect configuration

Configure:

- Number of masters/slaves

- Address decoding

- Arbitration policies

- QoS rules

📌 This is the heart of integration.

4️⃣ Subsystem integration

Each subsystem may internally contain:

- Registers

- Local bus (APB)

- Interrupt outputs

Example:

AXI → APB bridge → UART registers

📌 DV checks protocol correctness.

5️⃣ Clock & reset integration

Define:

- Clock domains

- Reset sources

- Reset ordering

📌 Many SoC bugs come from reset sequencing.

6️⃣ Interrupt integration

Connect:

- Peripheral interrupts

- DMA interrupts

- Error interrupts
to interrupt controller (GIC / INTC)

📌 Verify priority, masking, routing.

7️⃣ Power & low-power hooks

(Optional but common)

- Power domains

- Isolation signals

- Clock gating

📌 DV verifies state retention & wakeup.

8️⃣ SoC top-level assembly

In soc_top.v:

- Instantiate everything

- Connect all interfaces

- Tie off unused ports

📌 This is final RTL integration.

9️⃣ SoC-level verification

Run:

- Smoke tests

- Boot tests

- Stress traffic

- Error injection

📌 This is where DV spends most time.

🔟 Software bring-up (post-integration)

- Bare-metal tests

- OS boot

- Driver validation

📌 Confirms integration correctness.

🧠 One-screen mental flow

Architecture

↓

Address Map

↓

Interconnect Setup

↓

Subsystems

↓

Clock/Reset

↓

Interrupts

↓

soc_top.v

↓

SoC Verification

🎯 Verification engineer focus (important)

You mainly care about steps 2 → 9.

You do not redesign CPU or IP internals.

🔑 Interview one-liner

“SoC integration involves defining the architecture, address map, interconnect configuration, clock/reset and interrupt wiring, assembling everything in soc_top, and then verifying system-level behavior.”

If you want next, I can:

- Map these steps to real RTL files

- Show integration checklist

- Show typical SoC integration bugs

Just tell me 👌

Question:- how cpu  /subsyetem  select ip inside susb system?

Ans:-

Absolutely 👍 — let’s make this very concrete with a clean, realistic example.
I’ll show 5 subsystems, how their address ranges are defined, and who uses that information.

🧱 Step 1: Define SoC address space (example)

Assume a 32-bit address space.

We divide it into big chunks, one per subsystem.

🧩 Step 2: Address map for 5 subsystems

| Subsystem | Base Address | Size | Address Range |
| --- | --- | --- | --- |
| Boot ROM | 0x0000_0000 | 256 KB | 0x0000_0000 – 0x0003_FFFF |
| Memory Subsystem | 0x8000_0000 | 16 MB | 0x8000_0000 – 0x80FF_FFFF |
| Peripheral Subsystem | 0x4000_0000 | 1 MB | 0x4000_0000 – 0x400F_FFFF |
| DMA Subsystem | 0x5000_0000 | 1 MB | 0x5000_0000 – 0x500F_FFFF |
| Debug / System Ctrl | 0xF000_0000 | 1 MB | 0xF000_0000 – 0xF00F_FFFF |

👉 This table comes from the SoC architecture spec.

🧠 Step 3: How this is used in hardware (important)

❌ Not inside CPU

CPU does not contain this table.

✅ Inside interconnect (Level-1 decode)

Pseudo RTL in AXI interconnect:

if (addr >= 32'h0000_0000 && addr <= 32'h0003_FFFF)

sel_bootrom = 1;

else if (addr >= 32'h4000_0000 && addr <= 32'h400F_FFFF)

sel_periph_ss = 1;

else if (addr >= 32'h5000_0000 && addr <= 32'h500F_FFFF)

sel_dma_ss = 1;

else if (addr >= 32'h8000_0000 && addr <= 32'h80FF_FFFF)

sel_mem_ss = 1;

else if (addr >= 32'hF000_0000 && addr <= 32'hF00F_FFFF)

sel_sysctrl = 1;

else

sel_default = 1; // error slave

This logic is auto-generated or configured in real projects.

🧩 Step 4: What software sees (important distinction)

Software uses the same map:

#define UART0_BASE   0x40000000

#define DMA_BASE     0x50000000

#define DDR_BASE     0x80000000

But software does not control selection — hardware does.

🧠 Step 5: Inside one subsystem (example)

Peripheral Subsystem internal map

0x4000_0000 – 0x4000_0FFF → UART

0x4000_1000 – 0x4000_1FFF → SPI

0x4000_2000 – 0x4000_2FFF → I2C

Local decode:

local_addr = addr - 32'h4000_0000;

if (local_addr < 32'h1000)

sel_uart = 1;

else if (local_addr < 32'h2000)

sel_spi = 1;

else

sel_i2c = 1;

🧪 Verification engineer checklist

You verify:

- No address overlap

- Correct routing to subsystem

- Correct routing inside subsystem

- Boundary address behavior

- Error response for invalid ranges

🎯 Interview one-liner

“Subsystem address ranges are defined in the SoC address map and implemented in the interconnect, with each subsystem further decoding its own internal address space.”

🔥 Final takeaway (remember forever)

Big ranges → subsystems
Small ranges → IPs inside subsystems

If you want next:

- Draw address map diagram

- Show linker script relation

- Show assertions for address decode

Just say 👍

Question:- how select ip in soc  with example?

Yes — you are on the right track! Let me explain step by step, using your example, in a clear SoC integration context.

1️⃣ CPU generates an address

- CPU generates a memory-mapped address to access some register or memory.

- Example: CPU wants to write to 0x0000_0010.

2️⃣ Interconnect checks which subsystem it belongs to

- The SoC interconnect (like AXI interconnect or bus matrix) has a map of address ranges for each subsystem.

- Example:

| Subsystem | Address Range |
| --- | --- |
| Subsystem 0 | 0x0000_0000 – 0x0000_0019 |
| Subsystem 1 | 0x0000_0020 – 0x0000_0030 |
| Subsystem 2 | 0x0000_0031 – 0x0000_0040 |

- The interconnect decodes the CPU address and routes the request to the correct subsystem.

✅ So if CPU generates 0x0000_0015, interconnect sees it is within Subsystem 0 range and forwards the request there.

3️⃣ Inside the subsystem, find the target IP

- Each subsystem itself may contain multiple IP blocks, each mapped to smaller sub-ranges.

- Example inside Subsystem 0:

| IP | Local Address Range |
| --- | --- |
| IP-A | 0x0000 – 0x0009 |
| IP-B | 0x000A – 0x0019 |

- Subsystem logic subtracts its base address and finds which IP gets the transaction.

- Continuing example:

  - CPU sends 0x0000_0015

  - Subsystem 0 base = 0x0000_0000

  - Local address = 0x0015 – 0x0000 = 0x0015

  - This falls in IP-B’s range → routed to IP-B.

4️⃣ So the overall flow is:

CPU generates address → Interconnect decodes subsystem → Subsystem decodes IP → Target IP executes operation

✅ Key points

- Interconnect only routes between subsystems, not individual IPs.

- Subsystems maintain internal mapping for IPs.

- Address ranges must not overlap; otherwise you get conflicts.

- The CPU doesn’t need to know which IP; it just uses memory-mapped addresses.

If you want, I can draw a small diagram with CPU → Interconnect → Subsystem → IP mapping showing the ranges you mentioned. It makes this concept much easier to visualize.

Do you want me to draw it?

OTHER_EXAMPLE:-

Absolutely! Let’s go through another concrete example with numbers so it’s easier to visualize.

Suppose we have 3 subsystems in an SoC:

| Subsystem | Address Range (hex) |
| --- | --- |
| Subsystem 0 (SS0) | 0x0000_0000 – 0x0000_00FF |
| Subsystem 1 (SS1) | 0x0000_0100 – 0x0000_01FF |
| Subsystem 2 (SS2) | 0x0000_0200 – 0x0000_02FF |

Step 1: CPU generates an address

- CPU wants to write to 0x0000_0125.

Step 2: Interconnect decodes subsystem

- Interconnect compares the address with the subsystem ranges:

0x0000_0000 – 0x0000_00FF → SS0

0x0000_0100 – 0x0000_01FF → SS1 ✅

0x0000_0200 – 0x0000_02FF → SS2

- So the interconnect forwards this transaction to Subsystem 1.

Step 3: Subsystem decodes target IP

- Subsystem 1 contains 2 IP blocks:

| IP | Local Address Range (hex) |
| --- | --- |
| IP-A | 0x0000 – 0x007F |
| IP-B | 0x0080 – 0x00FF |

- Subsystem 1 base = 0x0100

- Local address inside subsystem = 0x0125 – 0x0100 = 0x0025

- 0x0025 falls inside IP-A’s local range → Subsystem forwards transaction to IP-A.

Step 4: IP executes the operation

- IP-A receives the request, does the read/write, and returns the response back through the same path:

IP-A → Subsystem 1 → Interconnect → CPU

✅ Notes:

- CPU only sees a single memory map; it doesn’t care about subsystems or IPs.

- Interconnect only decides which subsystem gets the request.

- Subsystem decides which IP handles it.

- This allows hierarchical mapping, e.g., SoC with dozens of subsystems, each with dozens of IPs.

If you want, I can make a diagram with color-coded address ranges for CPU → Interconnect → Subsystem → IP. That makes it super easy to visualize.

Do you want me to do that?

SOC_INTEGRATION_FLOW

Ah! I see the diagram you uploaded — it’s essentially a SoC top-level integration flow from an IP-centric perspective. Let me break down each block with simple explanations, examples, and why/when we need them.

1️⃣ IP Selection

- What: Decide which IPs (Intellectual Property blocks) you need in your SoC.

- Why: SoC design often involves combining multiple functional blocks (CPU, DMA, UART, GPIO, etc.). You only pick the ones needed for your SoC use-case.

- Example:

  - Designing a multimedia SoC: CPU + GPU + DDR controller + Video decoder.

  - Don’t include USB if it’s not needed.

2️⃣ IP Configuration

- What: Configure each IP’s internal settings according to your SoC requirements.

- Why: IPs often have configurable parameters like data width, number of channels, clock speed, or buffer size.

- Example:

  - DMA IP: Configure max transfer size = 256 bytes.

  - UART IP: Configure baud rate = 115200, parity = none.

3️⃣ Memory Map

- What: Define the address range where each IP will live in the CPU’s address space.

- Why: CPU accesses peripherals through memory-mapped addresses. Correct mapping ensures CPU reads/writes reach the right IP.

- Example:

| IP | Address Range |
| --- | --- |
| UART0 | 0x4000_0000 – 0x4000_00FF |
| GPIO0 | 0x4000_0100 – 0x4000_01FF |
| DMA | 0x4000_0200 – 0x4000_02FF |

4️⃣ Glue Logic Development

- What: Logic that connects IPs to the interconnect or bus (AXI/AHB/APB).

- Why: Different IPs may have different interface protocols, widths, or clock domains. Glue logic ensures they talk correctly.

- Example:

  - A 32-bit peripheral connected to a 64-bit AXI bus. You need width adaptation logic.

  - Converting AHB protocol signals to APB for a legacy IP.

5️⃣ Reset and Clock Generation Logic

- What: Provide proper reset signals and clock signals to IPs.

- Why: IPs need synchronized clocks and proper reset at startup for deterministic behavior.

- Example:

  - CPU runs at 1 GHz, peripheral bus at 100 MHz → need clock divider/multiplexer.

  - Reset logic ensures peripherals start in a known state.

6️⃣ IP Integration with AXI/AHB System Bus

- What: Physically connect IPs to the system interconnect or bus.

- Why: SoC communication happens over a bus or crossbar; this step ensures each IP is accessible.

- Example:

  - CPU writes 0x4000_0105 → bus routes this to GPIO0 → GPIO pin toggles.

7️⃣ IO (Input/Output) Pad Files

- What: Define actual pins on the chip that connect to the outside world.

- Why: Internal IP signals need physical connections to chip pins (pads) for communication with other devices.

- Example:

  - UART TX → IO pin 12

  - GPIO → IO pins 20-27

8️⃣ Top Level File

- What: A single RTL module that instantiates all IPs, interconnect, clock/reset logic, and IO pads.

- Why: This is the final “SoC blueprint” that will be synthesized and verified.

- Example:

module top_soc(

input clk,

input rst_n,

output uart_tx,

input uart_rx,

inout [7:0] gpio

);

wire [31:0] axi_data;

cpu u_cpu(...);

dma u_dma(...);

gpio u_gpio(.gpio_pins(gpio), ...);

uart u_uart(.tx(uart_tx), .rx(uart_rx), ...);

axi_interconnect u_axi(...);

endmodule

✅ Summary: Why/When

| Step | Why Important | When Used |
| --- | --- | --- |
| IP Selection | Only include needed functionality | Always at project start |
| IP Configuration | Tailor IP to SoC requirements | After selection, before integration |
| Memory Map | CPU knows where each IP lives | Before integration and software bring-up |
| Glue Logic | Connect incompatible interfaces | During integration of multiple IPs |
| Reset/Clock | Proper startup & timing | Always needed |
| Bus Integration | Ensure CPU & DMA can access IPs | Integration phase |
| IO Pads | Physical chip connectivity | Pre-synthesis & layout |
| Top-Level File | Complete RTL for synthesis | Final RTL integration |

If you want, I can also draw a simple hierarchical diagram showing CPU → Interconnect → Subsystem → IP → IO, mapping this flow with addresses like we discussed earlier. It makes this much easier to understand visually.

Do you want me to make that diagram?

You’re asking about the SoC integration flow, which is essentially the step-by-step process followed to take all IP blocks, subsystems, and interfaces and integrate them into a complete System-on-Chip. I’ll explain it in a clear flow, from design to verification, with real-world context.

1. IP Collection and Verification

- Goal: Ensure all IP blocks are ready for integration.

- Steps:

  - Collect all IPs (processor, memory controllers, peripherals, etc.).

  - Check each IP individually using simulation/testbench.

  - Verify IP meets specs (timing, protocol compliance, reset behavior).

- Example: AXI IP verified for read/write before SoC integration.

2. Define SoC Architecture

- Goal: Plan address maps, clocks, resets, and interconnects.

- Steps:

  - Draw block diagram of CPU, memory, peripherals, and interconnect.

  - Assign address map for each subsystem.

    - Subsystem 1 → 0x0000_0000 to 0x0FFF_FFFF

    - Subsystem 2 → 0x1000_0000 to 0x1FFF_FFFF

  - Plan clocks, resets, power domains.

- Example: CPU accesses DMA at 0x2000_0000; interconnect routes it correctly.

3. Interconnect and Bus Integration

- Goal: Connect all IPs with proper interconnect (like AXI/AMBA/AHB).

- Steps:

  - Connect masters (CPU, DMA) and slaves (memory, peripherals) via interconnect.

  - Configure address decoding for each IP/subsystem.

  - Configure arbitration and QoS if multiple masters exist.

- Example: AXI crossbar connecting CPU, GPU, and memory with arbitration.

4. Reset and Clock Tree Integration

- Goal: Ensure all IPs get proper clocks and resets.

- Steps:

  - Generate/reset signals for all IPs.

  - Check reset sequencing.

  - Make sure clock domains are correctly connected.

- Example: CPU and peripherals in separate clock domains with proper synchronizers.

5. Interrupt Integration

- Goal: Centralize interrupts for CPU handling.

- Steps:

  - Connect peripheral interrupt lines to Interrupt Controller (INTC).

  - Map interrupts to CPU vectors.

  - Check priority and masking.

- Example: DMA done interrupt goes to INTC → CPU services the DMA completion.

6. Power & Low-Power Integration

- Goal: Implement power domains and gating.

- Steps:

  - Assign power domains to IPs.

  - Insert power switches, isolation, and retention cells.

  - Check sequences for power-up and power-down.

- Example: GPU powers down when not used; memory retains state.

7. Software and Driver Integration

- Goal: Ensure CPU can boot and access IPs correctly.

- Steps:

  - Load bootloader and OS.

  - Test memory map access.

  - Test peripheral drivers (DMA, UART, timers).

- Example: CPU programs DMA source/dest and triggers transfer successfully.

8. Verification and Validation

- Goal: Ensure SoC works as intended.

- Steps:

  - Run RTL simulation with SoC-level testbench.

  - Functional verification:

    - CPU boot

    - Memory read/write

    - Interrupt handling

    - DMA transfers

  - Run integration-specific scenarios, including:

    - Clock domain crossing

    - Reset sequences

    - Address decoding errors

- Example: CPU writes to memory via DMA → memory receives correct data → interrupt triggers.

9. Timing and Physical Integration

- Goal: Ensure SoC meets timing for synthesis and layout.

- Steps:

  - Synthesize integrated RTL.

  - Run timing analysis (STA) for all paths.

  - Insert buffers or adjust clocks as needed.

- Example: AXI bus meets timing for 400 MHz operation.

10. Tape-Out / Silicon Validation

- Goal: Move integrated design to silicon and verify real hardware.

- Steps:

  - Fabricate SoC.

  - Bring-up on board.

  - Run test programs to check real hardware integration.

- Example: Run memory stress test, DMA transfers, peripheral interrupts.

Summary Flow (High-level)

IP Verification → Architecture Definition → Interconnect Integration → Reset/Clock Setup

→ Interrupt Integration → Power/Low-power Setup → Software/Driver Integration

→ Functional Verification → Timing/Physical Checks → Tape-out / Silicon Validation

If you want, I can draw a clean SoC integration flow diagram showing CPU, interconnect, memory, peripherals, DMA, interrupts, and clocks. It will make this flow much easier to visualize.

Do you want me to do that?

++++++++++++++++++++++++++SOC_Integration+++++++++++++++++++++++++++++++

Thanks for sharing the diagram! I can help clarify it and expand on the SoC integration flow shown in your image. Let’s break it down step by step based on what you wrote:

1. IP Selection

- What it means: Decide which IP blocks are needed for your SoC based on system requirements.

- Example:

  - CPU core → e.g., Arm Cortex-M7

  - Memory controllers → DDR, SRAM

  - Peripherals → UART, GPIO, Timer, DMA

- Key considerations:

  - Functionality

  - Protocol support (AXI, APB, SPI, etc.)

  - Resource usage (area, power)

2. IP Configuration

- What it means: Set up each IP for integration.

- Includes:

  - Addressable size (e.g., 32 KB or 64 KB for a peripheral)

  - Protocol settings (AXI width, burst mode)

  - Interrupt enable/disable

- Example:

  - DMA: configure source/destination width, burst length

  - UART: configure baud rate and buffer size

3. Memory Map

- What it means: Assign address ranges to each IP/subsystem in the SoC.

- Steps:

  - Decide base and limit addresses for each subsystem.

  - Ensure no overlap between IPs.

  - Communicate address map to CPU and interconnect.

- Example:

| Subsystem | Base Address | Limit Address |
| --- | --- | --- |
| CPU | 0x0000_0000 | 0x0FFF_FFFF |
| DDR Ctrl | 0x1000_0000 | 0x1FFF_FFFF |
| UART | 0x2000_0000 | 0x2000_0FFF |
| DMA | 0x3000_0000 | 0x3000_0FFF |

4. Verification

- After selection, configuration, and memory mapping:

  - Simulate IP-to-IP communication.

  - Check CPU can access peripherals correctly.

  - Validate interrupts, DMA transfers, and clock/reset behavior.

💡 Tip: The first three steps—IP Selection → IP Configuration → Memory Map—are the foundation. Once these are correct, the SoC integration becomes much smoother.

+++++++++++++++++ Glue Logic development+++++++++++++++++++++++

Glue logic provides a common, well-defined platform (rules + logic) so that CPU, interconnect, subsystems, and IPs can all communicate correctly and consistently.

real SoC has multiple glue-logic blocks, not just one.

It is a set of rules + adapters + control logic that make different blocks behave as if they are on one unified system.

It ensures everyone agrees on:

- Who talks to whom

- How they talk

- When they talk

- What happens if two talk at once

Question:- “If interconnect already does routing, arbitration, etc., why do we still need glue logic?”

Absolutely! Let’s break down the purpose of Glue Logic in an SoC clearly, and I’ll give a realistic example so it’s easy to understand.

1. Purpose of Glue Logic in SoC

Glue Logic is needed whenever multiple IP blocks, subsystems, or interfaces need to work together but cannot directly communicate because of differences in:

- Protocols (e.g., CPU uses AXI, a peripheral uses APB)

- Signal widths (e.g., CPU is 64-bit, peripheral is 32-bit)

- Addressing (multiple IPs share the same interconnect)

- Clock domains (CPU runs at one frequency, peripherals at another)

- Interrupt sources (many peripherals need to send signals to CPU)

Essentially, Glue Logic acts as:

- Address Decoder & Router – decides which subsystem/IP should respond to a given CPU/DMA address.

- Bus Protocol Adapter – converts one bus type to another (AXI ↔ APB, AXI4 ↔ AXI4-Lite).

- Interrupt Controller – merges multiple interrupt signals and prioritizes them before sending to CPU.

- Arbitration Unit – decides which master (CPU, DMA, etc.) gets access to a shared bus.

- Clock & Reset Synchronizer – ensures signals from different clock domains work correctly.

In short: Glue Logic “glues” everything together so all parts of the SoC work correctly and efficiently.

2. Example Scenario

Let’s say we have a simple SoC with:

- CPU (AXI master)

- DMA controller (AXI master)

- 5 Subsystems, each with multiple peripherals:

  - Subsystem1: UART (APB), Timer (APB)

  - Subsystem2: SPI (APB), I2C (APB)

  - Subsystem3: GPU (AXI), Memory Controller (AXI)

  - Subsystem4: Audio (AXI), Camera (AXI)

  - Subsystem5: GPIO (APB), ADC (APB)

Without Glue Logic:

- CPU tries to access UART → CPU is AXI, UART is APB → incompatible, transaction fails.

- DMA wants memory transfer → arbitration with CPU not handled → bus contention.

- Multiple peripherals generate interrupts → CPU receives random signals → cannot decide priority.

With Glue Logic:

- Address Decoder:

  - CPU address 0x1000_0000 → UART → Glue Logic sends transaction to Subsystem1’s UART.

  - CPU address 0x2000_0000 → GPU → Glue Logic routes to Subsystem3’s GPU.

- Bus Protocol Converter:

  - CPU (AXI) → UART (APB) → Glue Logic converts AXI transaction to APB transaction.

- Arbitration Unit:

  - CPU and DMA both request memory → Glue Logic grants access to CPU first, DMA waits.

- Interrupt Controller:

  - Timer, UART, GPIO generate interrupts → Glue Logic merges and prioritizes → sends single interrupt line to CPU.

Result: All masters and peripherals communicate correctly, safely, and efficiently.

3. Why Glue Logic is Needed

- Integration of heterogeneous IPs → ensures different buses and protocols talk to each other.

- Conflict management → handles multiple masters requesting the same resource.

- System scalability → adding/removing IPs only needs updates in Glue Logic, not the whole CPU.

- Simplifies verification → instead of verifying every master/peripheral interaction, you verify the Glue Logic once.

Analogy:

Think of Glue Logic as a receptionist in a big office building:

- People (CPU, DMA) come in with letters (data/requests).

- The receptionist checks which department (subsystem/IP) the letter is for → forwards it correctly.

- If multiple people arrive at the same time, the receptionist decides who goes first.

- If the letter is in a foreign language (bus protocol difference), the receptionist translates it.

- If many alarms go off (interrupts), receptionist decides which one is critical and alerts management (CPU).

Without this receptionist → chaos! Letters lost, wrong departments contacted, alarms ignored.

If you want, I can draw a simplified SoC transaction flow diagram showing exactly how Glue Logic routes CPU/DMA transactions and interrupts—that makes it super easy to visualize.

Do you want me to do that?

🔥 Excellent question. This is the exact confusion everyone has at first — and you’re thinking in the right direction.

You’re basically asking:

“If interconnect already does routing, arbitration, etc., why do we still need glue logic?”

Let’s clear this once and for all, cleanly and correctly.

Short, crisp answer

👉 Interconnect is a generic highway.
Glue logic is the local traffic police + translator + rule enforcer.

They are not the same, and they solve different layers of the problem.

What the interconnect actually does (and what it does NOT)

Interconnect (AXI Interconnect / NoC) is responsible for:

✅ Moving transactions from master to slave
✅ Basic address-based routing
✅ AXI-compliant arbitration
✅ Data channel handling

That’s it.

It assumes:

- Everyone already speaks AXI

- Address maps are clean

- No protocol mismatch

- No SoC-specific behavior

What interconnect does NOT do ❌

Interconnect does NOT:

- Convert AXI ↔ APB

- Aggregate interrupts

- Handle reset sequencing

- Do power-domain logic

- Handle legacy IP quirks

- Implement SoC-specific rules

- Add security/firewall logic

- Fix protocol violations of IPs

👉 And this is where glue logic comes in.

Think in layers (this is the key mental model)

Application / Software View

---------------------------

Glue Logic  ← SoC-specific intelligence

---------------------------

Interconnect ← Generic transport fabric

---------------------------

Physical IP connections

Concrete example (real SoC case)

Scenario

- CPU + DMA = AXI masters

- UART, GPIO, TIMER = APB slaves

- AXI Interconnect in the SoC

Question:

Can AXI interconnect talk directly to UART?

❌ NO

Because:

- UART does not speak AXI

- Interconnect cannot translate protocols

Solution:

👉 Insert glue logic (AXI-to-APB bridge)

CPU ──AXI──> Interconnect ──AXI──> Glue Logic ──APB──> UART

Interconnect alone is useless here.

Another example: interrupts (very important)

Reality:

- 100 peripherals

- Each has 1 interrupt

CPU expects:

- Maybe 8 or 16 interrupt lines

Interconnect:

❌ Does not handle interrupts

Glue logic:

✅ Collects 100 interrupts
✅ Prioritizes
✅ Masks
✅ Routes to CPU

Without glue logic → CPU is blind.

Arbitration confusion (important distinction)

| Arbitration Type | Who does it |
| --- | --- |
| AXI channel arbitration | Interconnect |
| CPU vs DMA policy | Glue logic |
| Peripheral priority | Glue logic |
| QoS tuning | Glue logic |

Interconnect arbitrates signals
Glue logic arbitrates system intent

Why SoC teams NEVER rely only on interconnect

Because:

- IPs come from different vendors

- Some IPs are buggy or legacy

- Some IPs are non-AXI

- Power/reset requirements differ

- Security rules differ per chip

👉 Interconnect is too generic
👉 Glue logic is SoC-aware

One-line killer explanation (use this)

Interconnect moves transactions; glue logic makes those transactions meaningful and safe for the SoC.

Final clarity table

| Feature | Interconnect | Glue Logic |
| --- | --- | --- |
| AXI routing | ✅ | ❌ |
| AXI ↔ APB conversion | ❌ | ✅ |
| Interrupt handling | ❌ | ✅ |
| Reset sequencing | ❌ | ✅ |
| Power management | ❌ | ✅ |
| SoC-specific rules | ❌ | ✅ |
| IP quirks handling | ❌ | ✅ |

Bottom line (no confusion now)

✔ Interconnect is necessary but not sufficient
✔ Glue logic is mandatory in real SoCs
✔ They complement, not replace, each other

If you want, next I can:

- Walk through CPU → interconnect → glue → IP signal-level flow

- Explain what part verification teams verify vs interconnect VIP

- Show why most SoC bugs are glue-logic bugs

Just say the word 🙂

Yes 👍 — you’re absolutely right, and this is a very sharp observation.

Interconnect does decode addresses to decide which subsystem / slave port a transaction should go to.

So the real question you’re asking is:

“If interconnect already decodes addresses and routes to subsystems, why do we still need glue logic?”

Let’s answer this precisely, without hand-waving.

What address decode in interconnect REALLY means

Interconnect address decode = coarse-grain decode

It answers only this question:

“Which slave port should receive this transaction?”

Example:

0x4000_0000 – 0x4FFF_FFFF → Subsystem A

0x5000_0000 – 0x5FFF_FFFF → Subsystem B

That’s it.

Once it chooses the slave port, its job is done.

What interconnect does NOT do (critical)

After routing to a slave port, the interconnect:

❌ Does NOT know:

- How many IPs exist inside the subsystem

- Which IP should respond

- If protocol conversion is needed

- How interrupts work

- Reset / power state of the IPs

👉 That intelligence is outside the scope of interconnect.

Where glue logic takes over (fine-grain decode)

Once the transaction reaches the subsystem:

AXI Interconnect

|

|  (Address already routed to Subsystem A)

v

[ Subsystem Glue Logic ]

Now glue logic answers:

“Which IP inside this subsystem owns this address?”

Example inside Subsystem A:

0x4000_0000 – 0x4000_FFFF → UART

0x4001_0000 – 0x4001_FFFF → GPIO

0x4002_0000 – 0x4002_FFFF → TIMER

Interconnect will never do this.

Simple visual (very important)

CPU

|

AXI Interconnect

|  ← coarse decode (subsystem level)

|

Subsystem A

|

Glue Logic

|  ← fine decode (IP level)

|

IPs

Why this separation exists (by design)

1️⃣ Scalability

If interconnect decoded every IP:

- Interconnect config explodes

- Any IP change = interconnect change

Bad design.

2️⃣ Reusability

Subsystems are reused across chips.
Glue logic stays with the subsystem.
Interconnect stays generic.

3️⃣ Protocol isolation

Interconnect assumes AXI.
Glue logic handles AXI → APB / custom.

4️⃣ Ownership

- Interconnect = platform team / vendor IP

- Glue logic = SoC integration team

Clear responsibility boundary.

Real-world analogy (locks it in)

Think of an apartment building 🏢

- Interconnect = security at the main gate
→ “This visitor goes to Building A”

- Glue logic = receptionist inside the building
→ “This visitor goes to Flat 304”

Security does not care about flats.
Receptionist does not manage the whole city.

One-line killer answer (use this confidently)

Interconnect performs coarse address decoding to select a subsystem, while glue logic performs fine-grain decoding, protocol adaptation, and control inside the subsystem.

Final truth (no contradiction)

✔ Interconnect DOES decode addresses
✔ Glue logic ALSO decodes addresses
✔ They operate at different levels and responsibilities

Not duplication — hierarchy.

If you want, next I can:

- Show real AXI interconnect config vs glue RTL

- Walk through exact signal flow for one address

- Explain how verification tests both layers

Just say 👍