SENERIO_BASE_QUESTION_PCIE

1. Explain the complete flow of a TLP from transmitter to receiver including:

- Sequence Number

- LCRC

- Replay Buffer

- ACK/NAK

- Replay Timer

2. Question 2 (Deeper)

Suppose:

- Transmitter sends TLP with Seq = 10

- Then sends Seq = 11

- Receiver receives only Seq = 11 (Seq = 10 was lost)

👉 What exactly happens at receiver side?
👉 What NAK value is sent?
👉 What does transmitter replay?
👉 From which sequence number retransmission starts?

Ans:- Good 👍 you are thinking in correct direction — but your answer still has one critical mistake.

This is exactly where interviewers trap candidates.

Let’s break it carefully.

🚨 Scenario Again

Transmitter sends:

- TLP Seq = 10

- TLP Seq = 11

Receiver receives:

- Only Seq = 11

- Seq = 10 is lost

❌ Mistake in Your Answer

You said:

receiver seq no == 11 then ack for this packet

⚠️ This is wrong.

Receiver will NOT ACK Seq = 11.

Why?

Because PCIe Data Link Layer requires in-order delivery.

If receiver is expecting Seq = 10 but gets Seq = 11:

→ It detects a sequence number mismatch
→ It does NOT pass Seq = 11 to TL
→ It does NOT ACK 11

✅ Correct Behaviour (Very Important)

At Receiver:

Expected sequence number = 10
Received = 11

👉 Sequence mismatch detected

Receiver will:

- Discard Seq = 11

- Send NAK indicating expected sequence number = 10

NAK contains:

“I am expecting sequence number 10”

✅ At Transmitter:

When NAK(10) is received:

- Transmitter replays starting from sequence 10

- It retransmits:

  - Seq 10

  - Seq 11

  - Seq 12… (if any)

⚠️ It replays ALL unacknowledged TLPs starting from NAK sequence number.

🔥 Important Rule

Receiver NEVER ACKs out-of-order TLPs.

This is strict rule in PCIe.

🔎 Deeper Understanding Check

Now let me increase difficulty.

Question 3 (Advanced)

Suppose:

- Transmitter sends Seq 100, 101, 102

- Receiver correctly receives 100

- Receiver correctly receives 101

- Receiver receives corrupted 102 (LCRC failure)

👉 What happens?
👉 What ACK/NAK value is sent?
👉 Does receiver deliver 100 and 101 to TL?
👉 What does transmitter replay?

Think carefully before answering.

This question separates beginner from experienced PCIe engineer.

Ans:- 🔥 Very good — this answer is much better.

You are now thinking like someone who has actually debugged replay logic.

But let’s refine it to make it interview-perfect.

✅ Scenario Recap

Transmitter sends:

- Seq 100

- Seq 101

- Seq 102

Receiver receives:

- 100 → correct

- 101 → correct

- 102 → LCRC error

✅ Correct Receiver Behavior

Step 1: Seq 100

- LCRC OK

- In-order

- Deliver to TL

- Update expected seq → 101

Step 2: Seq 101

- LCRC OK

- In-order

- Deliver to TL

- Update expected seq → 102

Step 3: Seq 102

- LCRC FAIL

Receiver:

- Discards TLP

- Sends NAK(102)

✔️ Yes, NAK carries the sequence number expected next

✅ ACK Behavior Clarification (Important)

You said:

receiver send ack for 100,101 and send nak for 102

⚠️ In PCIe, ACK/NAK are not per-packet responses.

Receiver may send:

- Cumulative ACKs

- Or standalone NAK

Meaning:

When 100 and 101 are accepted,
Receiver sends ACK indicating:

“I have successfully received up to 101”

Then NAK(102) is sent when error detected.

✅ Transmitter Behavior

Upon receiving:

- ACK up to 101
→ Remove 100 and 101 from Replay Buffer

- NAK(102)
→ Retransmit starting from 102

So replay only:

- 102

- and any unacknowledged after 102

✔️ You answered this correctly.

🔥 Now Let’s Go Harder

Now I want to test if you REALLY understand replay.

Question 4 (Tricky One)

Suppose:

Transmitter sends:

- Seq 200

- Seq 201

- Seq 202

Receiver receives:

- 200 OK

- 201 OK

- 202 OK

Receiver sends ACK (for 202)

But…

ACK gets LOST in link.

Transmitter does NOT receive ACK.

Replay Timer expires.

👉 What happens now?

- Will transmitter replay?

- What will receiver do when it sees duplicate packets?

- Will duplicate packets reach Transaction Layer?

Answer carefully.
This is very common real bug scenario in silicon.

Good 👍 you are thinking in correct direction (Replay Timer expiry → retransmission),
but your answer is incomplete and slightly incorrect.

This is a very important real silicon scenario.

Let’s go step-by-step carefully.

🔁 Scenario Recap

Transmitter sends:

- 200

- 201

- 202

Receiver:

- Receives all correctly

- Delivers to TL

- Sends cumulative ACK (up to 202)

But ACK gets lost in the link

Transmitter:

- Never receives ACK

- Replay Timer expires

✅ What Happens at Transmitter?

When Replay Timer expires:

👉 Transmitter assumes packets were lost
👉 It replays ALL unacknowledged TLPs

So it retransmits:

- 200

- 201

- 202

Even though receiver already received them.

This is correct behavior.

✅ Now Important Part (You Missed This)

What does receiver do when it sees duplicate packets?

Receiver is expecting:

Expected Sequence Number = 203

But it receives:

- 200 (duplicate)

Sequence mismatch!

Now this is where PCIe is smart.

🧠 Receiver Behavior for Duplicate TLP

Receiver checks:

If received sequence number < expected sequence number
→ It is a duplicate replay packet

Receiver will:

- Discard duplicate TLP

- NOT deliver to TL (very important)

- Send ACK again (cumulative ACK up to 202)

🚨 Very Important Rule

Duplicate replay packets NEVER reach Transaction Layer.

DLL filters them.

🔥 So Final Flow

Replay Timer expires →
Transmitter replays 200, 201, 202 →
Receiver detects duplicates →
Drops them →
Sends ACK(202) again →
Transmitter finally removes them from Replay Buffer

System continues normally.

❗ Mistake in Your Answer

You said:

regenerate 202 packet waiting for response

❌ It does NOT regenerate only 202.
It regenerates ALL unacknowledged TLPs (200,201,202).

Replay always starts from Oldest UnACKed sequence number, not last one.

🎯 Now Next Level Question (Interview Killer)

Suppose:

- Replay Timer expires

- Transmitter replays 200,201,202

- Meanwhile receiver had already moved to L0s (low power state)

👉 What happens?
👉 Will replay happen in L0s?
👉 How does LTSSM interact with Replay?

Answer carefully.

Now we are mixing:
Data Link Layer + LTSSM + Power Management.

This is senior-level PCIe question.

Good attempt 👍 — but this answer is too vague and technically incorrect for a 4.7-year PCIe engineer.

This is exactly the type of question where interviewers check if you truly understand LTSSM + Data Link interaction.

Let’s break it properly.

🔁 Scenario

- Replay Timer expires

- Transmitter needs to retransmit TLPs

- But link is in L0s (low power state)

❌ Issue in Your Answer

You said:

transmitter replay enter in l0 state then l0 check l0s state and we move back from here

This is not accurate.

Replay logic does NOT “enter L0”.
Replay happens only in L0 state.

Important rule:

👉 TLP transmission is allowed only in L0
👉 No TLP transmission in L0s, L1, L2

✅ Correct Behaviour (Very Important)

🔹 What is L0s?

L0s is a standby state:

- Transmitter electrical idle

- Fast exit possible

- Used for power saving

But:

⚠️ No packet transmission allowed in L0s.

🔹 So What Happens When Replay Timer Expires in L0s?

When Replay Timer expires:

Transmitter wants to retransmit.

But link is in L0s.

So:

- LTSSM automatically initiates Exit from L0s → back to L0

- Once link returns to L0

- Then Data Link Layer performs replay

Replay never happens inside L0s.

🔥 Important Architecture Concept

LTSSM controls physical link state.

Data Link Layer depends on LTSSM.

If link is not in L0:
→ DLL transmission is blocked.

💡 Clean Interview Answer

If interviewer asks:

What happens if Replay Timer expires in L0s?

You should answer:

- Replay requires active link (L0 state)

- If link is in L0s, LTSSM first transitions link back to L0

- After link becomes active, DLL initiates replay

- No TLP transmission occurs in L0s

🎯 Now Let’s Increase Difficulty Again

Suppose:

- Link is in L1 (not L0s)

- Replay Timer expires

- There are unacknowledged TLPs

👉 What happens now?
👉 Can replay happen immediately?
👉 What additional handshake is required?
👉 Is DLL active in L1?

Think carefully.

This question checks deep power management understanding.

You answer:

Replay cannot occur in L1 because packet transmission is not allowed. When Replay Timer expires, the Data Link Layer requests retransmission, but since the link is in L1, LTSSM must first exit L1 and transition back to L0. L1 exit requires handshake and PHY reactivation. Once the link is restored to L0, the Data Link Layer replays unacknowledged TLPs from the replay buffer using the same sequence numbers.

Question:- how can we back on l1 to l0 state ?

| Feature | L0s Exit | L1 Exit |
| --- | --- | --- |
| Clock removed | No | Yes (can be) |
| Recovery needed | No | Often yes |
| FTS used | No | Yes (typically) |
| Exit latency | Very small | Larger |

Suppose:

- Link enters L1

- There are still unacknowledged TLPs

- Replay Timer is running

👉 Should Replay Timer continue running in L1?
👉 Or should it be paused?
👉 Why?

You say:

- Replay Timer runs only in L0

- When link enters L1, timer is paused

- When link exits back to L0, timer resumes from remaining time

- This prevents unnecessary replay

Correct Answer

Replay Timer is paused in L1.

It must NOT continue running.

Why?

Think logically:

- In L1, Data Link Layer is inactive.

- No TLP or DLLP transmission allowed.

- No ACK/NAK can be exchanged.

- Link is in low power state intentionally.

If Replay Timer continues running:
→ It will expire unnecessarily.
→ As soon as link exits L1, replay storm may occur.
→ This creates unnecessary retransmissio

Now I Increase Difficulty Again

Suppose:

- Transmitter sends Seq 300

- Receiver sends ACK(300)

- ACK arrives corrupted (LCRC fail on DLLP)

👉 What happens now?
👉 Will transmitter remove packet from replay buffer?
👉 Will replay happen?
👉 How is DLLP error handled?

Answer carefully.

This question tests:
DLLP integrity + replay interaction.

You say:

- Transmitter detects LCRC error

- Discards corrupted ACK

- Does not remove packet from replay buffer

- Replay timer continues

- On timeout, packet is retransmitted

- Receiver discards duplicate and sends ACK again

- You answer:

- If ACK DLLP is corrupted, its LCRC check fails at transmitter and it is discarded. Since valid ACK is not received, Replay Timer eventually expires. The transmitter retransmits the unacknowledged TLP from replay buffer. The receiver detects duplicate sequence number and discards it, then sends ACK again. The transaction layer remains unaware.

Now on the wire:

- Bit error happens

- ACK DLLP LCRC becomes invalid

When transmitter receives this corrupted ACK:

👉 It checks DLLP LCRC
👉 LCRC fails

So:

Transmitter discards the corrupted ACK.

It is treated as if ACK was never received.

Suppose:

- Receiver sends ACK(400)

- ACK is valid

- But transmitter mistakenly removes wrong sequence from replay buffer due to design bug

👉 What kind of failure will you see in system?
👉 How will you debug it?
👉 What waveform signals will you check?

Good that you’re trying to think failure-level 👍
But your answer is not correct and also too generic for a 4.7-year PCIe engineer.

This is not immediately a “fatal error” or “link fail”.

This is a Data Link Layer replay buffer management bug — and it creates very specific symptoms.

Let’s break it down like real silicon debugging.

🔁 Scenario

- Receiver sends valid ACK(400)

- Transmitter receives it correctly

- But due to design bug:

  - Transmitter removes wrong entry from Replay Buffer

  - For example removes 401 instead of 400

  - Or clears entire buffer incorrectly

🚨 What Will Happen in System?

Let’s assume:

Replay buffer had:
400, 401, 402

ACK(400) received.

Correct behavior:
→ Remove 400
→ Keep 401, 402

Buggy behavior:
→ Removes 401 (wrong one)

Now replay buffer contains:
400, 402

But 400 was already acknowledged and removed incorrectly.

🔥 What Failure Will You See?

Eventually:

- Receiver will send NAK(401)

- Transmitter tries to replay 401

- But 401 is missing from replay buffer ❌

Now what?

👉 Transmitter cannot retransmit required TLP
👉 Protocol violation
👉 Replay logic stuck
👉 Eventually link goes to Recovery

This causes:

- Replay timeout continuously

- Replay storm

- LTSSM entering Recovery repeatedly

- Possible Surprise Down / Link Down

Not immediate fatal error — but link instability.

🧠 What Symptoms You Will Observe?

In waveform:

1️⃣ Replay Buffer content mismatch

Check:

- Replay buffer write pointer

- Replay buffer read pointer

- ACK sequence number

- Oldest unacknowledged sequence

2️⃣ DLLP Trace

Check:

- ACK received value

- Expected next sequence

- NAK generation

- Replay start sequence

3️⃣ LTSSM Behavior

Check:

- Is link moving to Recovery?

- Is it retraining repeatedly?

🎯 How To Debug (Interview-Ready Answer)

If interviewer asks:

How will you debug it?

You should say:

- Check ACK DLLP value in waveform

- Compare with replay buffer removal logic

- Verify oldest unacknowledged pointer update

- Check sequence number tracking logic

- Confirm replay buffer memory index

- Monitor replay timer behavior

- Check for replay request sequence mismatch

Very structured answer impresses interviewer.

🔥 Important Concept

Wrong replay buffer removal causes:

- Replay request for missing packet

- Deadlock in Data Link Layer

- Link Recovery state entry

- Eventually Link Down if unresolved

🎯 Now Next Question (Even Deeper)

Suppose:

Receiver sends cumulative ACK(500)

Transmitter has:
498, 499, 500 in replay buffer

👉 What exactly does cumulative ACK(500) mean?
👉 Which entries are removed?
👉 What happens if ACK(499) was never seen?

Explain carefully.

This checks if you understand cumulative ACK semantics properly.